CAPACITANCE METER using ESP32

AIM: To calculate the capacitance value and transfer it via WiFi to an Android App

COMPONENTS:

- Arduino IDE

- ESP32 WiFi module

- Jumper wires

- Bread board

THEORY:

ESP32 integrates Wi-Fi (2.4 GHz band) and Bluetooth 4.2 solutions on a single chip, along with dual high performance cores, Ultra Low Power co-processor and several peripherals. Powered by 40 nm technology, ESP32 provides a robust, highly integrated platform to meet the continuous demands for efficient power usage, compact design, security, high performance, and reliability. ESP32 can interface with other systems to provide Wi-Fi and Bluetooth functionality through its SPI /SDIO or l2C / UART interfaces.

Procedure for finding capacitance:

For smaller values of capacitances, the stray capacitances come into play as they are not negligible value comparatively.

As Esp32 has 12 bit resolution for ADC, hence 3.3V is equivalent to 4095. If the capacitance is too large, Then voltage across capacitor is greater than 4000, that is near to 3.3V. If capacitance is too less, then voltage is read across the capacitor and hence the required capacitance is calculated as follows:

Let Cx be the capacitance to be measured and Cs be the stray capacitance, then

As Q=CV, since charges are same for both capacitors,

Cx * (4095-Vcs) = Cs * Vcs

Cx = (Cs * Vcs)/(4095-Vcs),  where Vcs is the voltage across stray capacitance.

The stray capacitance is measured by taking some known standard capacitance.

Below given picture tells the stray capacitance value:

Hence the stray capacitance is 17 pF by taking reference value as 39 pF.

For Larger values of capacitances, the stray capacitance is negligible and hence the total voltage will be there across stray capacitance as charge is equal and hence V is inversely proportional to C. Hence, we use RC circuit by activating the internal pull up resistances. The method is as depicted below:

The time constant of a capacitor is defined as the time it takes for the voltage across the capacitor to reach 63.2% of its voltage when fully charged. Larger capacitors take longer to charge, and therefore have larger time constants. An Arduino can measure capacitance because the time a capacitor takes to charge is directly related to its capacitance by the next equation.

TC=RxC.TC is the time constant of the capacitor (in seconds).

R is the resistance of the circuit (in Ohms).

C is the capacitance of the capacitor (in Farads).

So what we are going to do is charge the capacitor through the internal resistor of esp32. Using the ADC of the arduino we can measure the voltage that the capacitor reached. We start counting the same and when the voltage reaches 63.2% of the full charge we stop the charge and calculate the capacitance. We can obtain the capacitance value because we know the resistance of the circuit, the measured voltage and the time that it took to reach 63.2% of the full charge. From the equation above we obtain C as:

C = TC / R

ESP32 measures the capacitance of the unknown capacitor by recording the time it takes for the capacitor to reach 63.2% of its voltage when fully charged, then dividing that value by the known resistance. Now we should be ready to set up the capacitance meters and start measuring some capacitors.

CODE FOR ESP32

#define BLYNK_PRINT Serial

#include <WiFi.h>

#include <WiFiClient.h>

#include <BlynkSimpleEsp32.h>

#include <ThingSpeak.h>

// You should get Auth Token in the Blynk App.

// Go to the Project Settings (nut icon).

char auth[] = "<Blynk auth code>";

unsigned long myChannelNumber = <Thingspeak channel ID>;

const char * myWriteAPIKey = "<Thingspek Write API Key";

// Your WiFi credentials.

// Set password to "" for open networks.

char ssid[] = "<your SSID>";

char pass[] = "<Password>";

const int OUT_PIN = 25;

const int IN_PIN = 33;

const float IN_STRAY_CAP_TO_GND = 24.48;

const float IN_CAP_TO_GND  = IN_STRAY_CAP_TO_GND;

const float R_PULLUP = 34.8;

const int MAX_ADC_VALUE = 4095;

float capacitance;

WiFiClient client;

//BlynkTimer timer;

// This function sends Arduino's up time every second to Virtual Pin (5).

// In the app, Widget's reading frequency should be set to PUSH. This means

// that you define how often to send data to Blynk App.

void cap()

{

// You can send any value at any time.

// Please don't send more that 10 values per second.

pinMode(IN_PIN, INPUT);

digitalWrite(OUT_PIN, HIGH);

int val = analogRead(IN_PIN);

Serial.println(val);

digitalWrite(OUT_PIN, LOW);

if (val < 4000)

{

pinMode(IN_PIN, OUTPUT);

capacitance = (float)val * IN_CAP_TO_GND / (float)(MAX_ADC_VALUE - val);

Serial.println("Scale:   1pF-1nF");

Serial.println(capacitance-6.2);

}

else

{

pinMode(IN_PIN, OUTPUT);

delay(1);

pinMode(OUT_PIN, INPUT_PULLUP);

unsigned long u1 = micros();

unsigned long t;

int digVal;

do

{

digVal = digitalRead(OUT_PIN);

unsigned long u2 = micros();

t = u2 > u1 ? u2 - u1 : u1 - u2;

}

while ((digVal < 1) && (t < 400000L));

pinMode(OUT_PIN, INPUT);

val = analogRead(OUT_PIN);

digitalWrite(IN_PIN, HIGH);

int dischargeTime = (int)(t / 1000L) * 5;

delay(dischargeTime);

pinMode(OUT_PIN, OUTPUT);

digitalWrite(OUT_PIN, LOW);

digitalWrite(IN_PIN, LOW);

float capacitance = -(float)t / R_PULLUP / log(1.0 - (float)val / (float)MAX_ADC_VALUE);

Serial.println("Scale:   1pF-1nF");

if (capacitance > 1000.0)

{

Serial.println(capacitance / 1000.0);

}

else

{

Serial.println(capacitance);

}

}

// while (micros() % 1000 != 0);

delay(2000);

//Blynk.virtualWrite(V5,String(capacitance-6.20)+"  pF");

}

void setup()

{

// Debug console

Serial.begin(9600);

pinMode(OUT_PIN, OUTPUT);

pinMode(IN_PIN, OUTPUT);

Blynk.begin(auth, ssid, pass);

// You can also specify server:

//Blynk.begin(auth, ssid, pass, "blynk-cloud.com", 80);

//Blynk.begin(auth, ssid, pass, IPAddress(192,168,1,100), 8080);

// Setup a function to be called every second

//timer.setInterval(1000L, cap);

ThingSpeak.begin(client);

}

void loop()

{

Blynk.run();

//timer.run(); // Initiates BlynkTimer

cap();

Blynk.virtualWrite(V5,String(capacitance-6.20)+"  pF");

ThingSpeak.writeField(myChannelNumber, 1, capacitance, myWriteAPIKey);

}

Method 1:

ESP32 and the Android app are connected to the same Wifi. The capacitance value is sent to local server with a local ip address. The android app made using MIT app inventor accesses the local server using the local address and receives the capacitance value over the same WiFi.

The disadvantage with this thing is that both the devices the android app and the esp32 module should be connected to the same network.

Method 2:

Using Blynk app:

- Create a New Project (new Auth Token will be sent to your email);

- Add Value Display Widget;

- Go to Widget Settings ;

- Set PIN to Virtual Pin V5;

- Set Frequency to 3 seconds;

It means that Blynk app will request the data from Virtual Pin V5 every 3 seconds. Virtual Pin is a concept invented by Blynk Inc. to provide exchange of any data between hardware and Blynk mobile app.

The block diagram above shows the implementation.Code  written in Arduino ide and uploaded to esp32 which gets connected to the Wifi and sends the measured capacitance value to a web server and the android app accceses the value using the server over any Wifi.

RESOURCES

- MIT App code

- MIT app inventor 2

- Blynk app

- Thingspeak

RESULT:  Measured the value of capacitance and transferred the data to android app over WiFi using Esp32 .